Packet Filtering and Analysis using Wireshark/Burp Suite

### Aim

To capture, filter, and analyze network packets using Wireshark on Kali Linux in VirtualBox.

### Tools Required

- Hardware: Laptop/PC with VirtualBox installed

- Software:

  - Kali Linux (running inside VirtualBox)

  - Wireshark Network Analyzer

  - Apache2 web server

  - PHP interpreter

  - Browser

### Procedure

- Configure VirtualBox Network for Packet Capture

  - Open VirtualBox → Kali VM → Settings → Network.

  - Set Adapter 1:

    - Attached to: Bridged Adapter

    - Promiscuous Mode: Allow All

    - Cable Connected: ✔️

  - Save settings.

- Start Kali Linux VM

  - Boot the Kali Linux VM in VirtualBox.

- Run Wireshark as Root

sudo wireshark

  - Select network interface eth0

- Start Packet Capture

  - Click on eth0 to begin capturing packets.

  - Generate some traffic (open websites, ping servers, or run curl commands).

- Apply Display Filters for Analysis
Examples:

  - DNS Packets:

dns

  - HTTPS Packets:

tls

  - Packets from specific IP:

ip.addr == 18.161.216.37

  - TCP Port 443:

tcp.port == 443

- Analyze Captured Packets

  - Select any packet and expand:

    - Ethernet Layer: Source & Destination MAC addresses

    - IP Layer: Source & Destination IP addresses

    - TCP/UDP Layer: Port numbers, flags (SYN, ACK)

    - Application Layer: Protocol details (DNS query, HTTP request, TLS handshake)

- Stop Capture and Save

  - Stop capture (red square button).

  - Save capture file in .pcap format for submission.

### Part B: Capturing Login Credentials over HTTP

Step 1 – Setup Apache Web Server

sudo service apache2 start

sudo mkdir -p /var/www/html/testlogin

cd /var/www/html/testlogin

Step 2 – Create Login HTML Page

sudo nano login.html

html

CopyEdit

<!DOCTYPE html>

<html>

<head>

<title>Test Login Page</title>

</head>

<body>

<h2>Login Form</h2>

<form action="login.php" method="post">

Username: <input type="text" name="username"><br><br>

Password: <input type="password" name="password"><br><br>

<input type="submit" value="Login">

</form>

</body>

</html>

Save (Ctrl + O → Enter → Ctrl + X).

Step 3 – Create PHP Script

sudo nano login.php

php

CopyEdit

<?php

$username = $_POST['username'];

$password = $_POST['password'];

echo "<h2>Login Attempt</h2>";

echo "Username: " . $username . "<br>";

echo "Password: " . $password . "<br>";

?>

Step 4 – Start/Enable Apache

sudo service apache2 status

sudo service apache2 start

Step 5 – Test Page

Browser:

http://127.0.0.1/testlogin/login.html

Or: Type in bash

curl http://127.0.0.1/testlogin/login.html

Step 6 – Capture with Wireshark

- Open Wireshark → select lo (loopback) interface.

- Start capture.

- Apply filter:

http.request.method == "POST"

Step 7 – Perform Login Attempt

- Username: admin

- Password: test123

- Submit form.

Step 8 – Analyze Packets

- Locate POST packet.

- Right-click → Follow → TCP Stream.

- Credentials will be visible in plain text.

## Observations

- Network traffic from various applications can be captured in real-time.

- Protocol filters help narrow down the traffic of interest.

- Details such as source/destination IP, MAC addresses, ports, and application data can be examined.

- HTTP POST requests transmit data in plaintext.

- Credentials can be directly read in the captured packets when using HTTP.

- Using HTTPS would encrypt this data and prevent such interception.

## Post-Lab Discussion

- Bridged Adapter mode allows the VM to be part of the physical network, enabling packet sniffing.

- Promiscuous mode is essential to capture packets not directly addressed to the capturing machine.

- HTTPS traffic is encrypted, so packet payloads are unreadable without decryption keys.

- HTTP lacks encryption, making it vulnerable to credential theft.

- Tools like Wireshark can be used by attackers to sniff sensitive data if encryption is not in place.

- Transitioning to HTTPS mitigates this risk by encrypting data in transit.

- The loopback interface captures traffic within the local system (127.0.0.1).

## Viva Questions

- What is the difference between a display filter and a capture filter in Wireshark?

- Why do we use promiscuous mode in packet capturing?

- How can you identify a TCP three-way handshake in Wireshark?

- What is the difference between HTTP and HTTPS in terms of packet analysis?

- Can Wireshark decrypt HTTPS traffic? Under what conditions?

- Why are credentials visible in HTTP packet captures but not in HTTPS?

- What is the significance of the loopback interface (lo) in Wireshark?

- How can a POST request be identified in Wireshark?

- What is the difference between GET and POST methods in HTTP?

- How does HTTPS protect against packet sniffing attacks?